# IoT Assignment – BHAMARE PIYUSH GIRISH

Date 02/07/2026

# Q1. Smart Irrigation — Arduino UNO vs ESP32: Which Would You Choose?

Choice: ESP32

- Connectivity requirement: The system must send data to a mobile app every 5 seconds — this requires internet/Wi-Fi connectivity, which ESP32 provides natively with its built-in Wi-Fi and Bluetooth module.

- Processing power: ESP32 has a dual-core 240 MHz processor, 520 KB RAM, and enough GPIO pins to handle the soil moisture sensor, pump relay, and cloud communication simultaneously without bottlenecks.

- Built-in connectivity: Arduino UNO has zero native wireless capability. Adding an ESP8266/Wi-Fi shield increases cost, complexity, and failure points, making the design messier and less reliable.

- Power efficiency: ESP32 supports deep sleep mode (~10 µA), critical for a battery-powered irrigation system running outdoors. Arduino UNO has no comparable sleep modes.

- Cost comparison: ESP32 (~Rs. 350) vs Arduino UNO (~Rs. 400) + Wi-Fi module (~Rs. 200) — ESP32 is actually cheaper and more capable for this IoT use case.

If internet is NOT required: Arduino UNO becomes the better choice — simpler, cheaper, lower power, no unnecessary Wi-Fi overhead, and perfectly capable of reading a soil moisture sensor and toggling a relay locally.

Conclusion: ESP32 for internet-connected irrigation; Arduino UNO for offline/local-only control systems.

# Q2. 500 Sensors Over 2 km — Wi-Fi vs Bluetooth vs LoRa vs MQTT

- Wi-Fi — not suitable alone: Range is only 50–100 m and power consumption is high (~150–300 mA active). Deploying Wi-Fi access points across a 2 km factory would be expensive and impractical for battery-powered sensors.

- Bluetooth — not suitable: Range is only ~10 m (BLE up to ~50 m) — far too short for a 2 km factory area. Suitable only for very local sensor clusters.

- LoRa — best physical protocol: Covers 2–15 km with very low power consumption (~10–20 mA transmitting). Each gateway can handle ~100–300 sensors — ideal for wide-area, battery-powered industrial sensor networks.

- MQTT — best application protocol: Not a wireless protocol; a lightweight publish-subscribe messaging protocol over TCP/IP with efficient 2-byte headers, perfect for gateway-to-cloud data transfer.

Best combination: LoRa + MQTT. Sensors transmit via LoRa to distributed gateways (one per ~200 sensors); each gateway forwards data to a central server via MQTT over Ethernet/cellular — giving full 2 km coverage, low power, reliable delivery, and easy cloud integration.

Deployment architecture: 5 LoRa gateways placed every 400 m across the factory, each collecting from ~100 nearby sensors and publishing to an MQTT broker that feeds a real-time dashboard.

Conclusion: LoRa for sensor-to-gateway communication + MQTT for gateway-to-server data transfer.

# Q3. Temperature Sensor Showing Impossible Values (–120°C or 300°C)

Diagnose in this systematic order — cheapest/easiest to most hardware-intensive:

- Check software first: Verify the raw-ADC-to-temperature formula, confirm the correct sensor library/model is selected, and check for unit mismatches (Celsius vs Fahrenheit vs Kelvin).

- Inspect wiring: Check for loose connections, broken wires, swapped VCC/GND, or incorrect pull-up resistors (DS18B20 needs 4.7 kΩ). A floating data line causes random extreme readings.

- Check power supply: Measure the actual supply voltage with a multimeter. Insufficient or noisy voltage (e.g. 3.0 V instead of 3.3 V) causes garbage output or failed initialization.

- Test the ADC: Feed a known stable voltage (e.g. exactly 1.65 V from a voltage divider) into the ADC pin and confirm it reads the expected digital value. A faulty ADC reads wrong values regardless of sensor.

- Replace the sensor: Substitute a known-working sensor. If readings normalize, the original sensor is faulty; if not, the problem is upstream (wiring/power/ADC).

Rule: Software → Wiring → Power Supply → ADC → Sensor — this order saves time and avoids unnecessary component replacement.

# Q4. ESP32 Weather Station — 6 Months on Battery: Modifications

- Deep sleep mode (most important): Wake every 5–10 minutes, read sensor, transmit, sleep again. Deep sleep draws ~10 µA vs ~240 mA active — extends battery life ~100x.

- Battery sizing: Use a large LiPo battery (10,000 mAh minimum) plus a small 5 W solar panel with a TP4056 charging module for self-sustaining outdoor operation.

- Low-power radio: Replace continuous Wi-Fi with LoRa (~20 mA transmitting vs ~240 mA for Wi-Fi); LoRa also has longer range.

- Low-power sensors: Use a BME280 (temperature + humidity + pressure, ~3.6 µA sleep current) instead of DHT11/DHT22, which have higher standby current.

- Disable unused peripherals: Turn off Bluetooth if unused (WiFi.mode(WIFI_STA) + btStop()), disable unused ADC/GPIO and the brown-out detector.

- Reduce transmission frequency: Weather changes slowly, so transmitting every 10 minutes instead of every 1 minute cuts radio-on time 10x with negligible data-quality loss.

- Watchdog timer: Add a hardware watchdog so the device auto-reboots if firmware hangs, preventing a frozen device from draining the battery.

Summary: Deep sleep + LoRa + solar charging + BME280 + reduced transmission frequency = well over 6 months on a single 10,000 mAh battery under typical conditions.

# Q5. Automatic Braking System — Communication with Near-Zero Delay

Choice: CAN Bus (Controller Area Network)

- Why CAN Bus: The industry-standard protocol for real-time, safety-critical automotive systems — up to 1 Mbps with deterministic microsecond latency and guaranteed delivery within known time bounds. Already used across all modern vehicles (ABS, airbags, engine control, stability systems), with built-in error detection, fault confinement, and a multi-master architecture (no single point of failure).

- Why not wireless (Wi-Fi/Bluetooth): Wireless introduces variable latency (1–50 ms), packet loss risk, and interference. A 10 ms delay at 100 km/h means the vehicle travels ~28 cm before braking begins — unacceptable.

- Why not Ethernet: Standard Ethernet is non-deterministic (collision delays). Automotive Ethernet with TSN (Time-Sensitive Networking) is used in newer vehicles, but CAN Bus remains standard for critical low-latency signals.

- For ultra-short distances (ECU-internal): SPI or direct GPIO hardware interrupts between chips within the same ECU give sub-microsecond response; CAN Bus handles inter-ECU communication across the vehicle.

Conclusion: CAN Bus for inter-ECU vehicle communication; direct GPIO/SPI for intra-ECU signals; never wireless for life-safety braking systems.

# Q6. Single Sensor Placement in a Classroom (Square Cuboid, side a, height b)

Optimal position: Geometric center horizontally (a/2, a/2), at height b/3 from the floor (~1.0–1.2 m) — the HVAC-standard occupancy zone that best represents human-experienced temperature.

- Why height b/3: Air temperature is not uniform vertically — warm air rises, cool air sinks. Floor level runs ~2–3°C cooler and the ceiling ~3–5°C warmer than the middle zone; b/3 corresponds to seated/breathing level.

- Why the geometric center: Farthest from all walls, windows, and doors, minimizing influence from boundary effects — solar radiation through windows, cold drafts from doors, heat from equipment near walls.

- Poor placement — near window: Direct solar radiation can raise readings 5–15°C above actual air temperature (measuring radiant heat, not air temperature).

- Poor placement — near door/vent: Drafts and HVAC airflow create localized pockets that don’t represent the room average, fluctuating 3–8°C with every door opening.

- Poor placement — near heat sources: Computers, projectors, and lights emit local heat; a sensor within 0.5 m can read 5–20°C above actual room temperature.

- Poor placement — near ceiling (height b): Thermal stratification concentrates warm air at the ceiling; a sensor there reads 3–5°C higher than what occupants experience, causing the HVAC system to under-heat the room.

# Q7. When Does an Analog Sensor Outperform a Digital Sensor?

- Continuous high-resolution measurement: Analog output is a smooth, continuous voltage with no quantization steps — important for audio, vibration, or fine pressure measurements where digital step-sampling would miss changes.

- Lower latency: Analog sensors respond instantaneously — no ADC conversion time or I2C/SPI overhead — capturing microsecond-scale changes in real time.

- High-frequency signals: When a signal changes faster than a digital sensor’s sampling rate, the digital sensor aliases the reading; an analog sensor with a fast external ADC avoids this.

- Cost and simplicity: For basic applications (light intensity, soil moisture, simple temperature), an analog sensor on the microcontroller’s built-in ADC is cheaper and simpler than a digital sensor requiring a communication protocol.

- Noisy communication environments: Digital lines (I2C, SPI, UART) can suffer corrupted packets in electrically noisy environments; an analog voltage is easier to filter and less prone to protocol errors.

- When the MCU already has a good ADC: A high-quality 12-bit/16-bit onboard ADC makes an added digital sensor’s internal conversion redundant — the analog sensor exploits the MCU’s own ADC directly.

Examples: analog microphones (audio capture), analog thermistors (fast thermal response), analog force-sensitive resistors (FSR), pH electrodes, analog accelerometers for high-frequency vibration analysis.

# Q8. Complete IoT Architecture — Block Diagram with Failure Points & Solutions

| Layer | Component | Failure Point | Solution |
| --- | --- | --- | --- |
| 1. Sensing | EC Probe + DS18B20 | Probe corrosion / loose wire → garbage readings | Use SS304 stainless-steel probes; add connection-integrity check in software |
| 2. Processing | ESP32 Microcontroller | Firmware crash / memory overflow halts operation | Hardware watchdog timer auto-reboots on stall |
| 3. Power Supply | 9V Battery + 7805 Regulator | Voltage drop under load causes sensor misbehavior | Add 100 µF decoupling capacitor at regulator output; use an LDO regulator |
| 4. Display | LCD I2C + LEDs + Buzzer | Wrong I2C address / loose SDA-SCL wire → blank display | Scan I2C bus on boot; try both 0x27 and 0x3F addresses |
| 5. Communication | ESP32 Wi-Fi | Network unavailable disrupts upload | Store results locally in SPIFFS, upload in batch on reconnect |
| 6. Cloud API | Telegram + Google Sheets | API rate limit or server downtime blocks uploads | Exponential backoff retry logic + message queue in firmware |
| 7. User Interface | Telegram App + Browser | App not installed / no mobile signal | SMS fallback via GSM module (SIM800L) |

Data flow: sensors (Layer 1) → ESP32 processing (2) → stable power (3) → local display (4) → Wi-Fi upload (5) → cloud storage/alerts (6) → operator’s phone/browser (7). Each arrow in the chain is a potential failure point, addressed individually above.

# Q9. Wi-Fi Attacker Gains Access — Possible Attacks & Security Improvements

Possible attacks:

- Data sniffing: Intercepting unencrypted sensor data in transit (EC values, timestamps, batch info) in plain text. Fix: use HTTPS (TLS) for all API calls, never plain HTTP.

- Man-in-the-middle (MITM): Attacker sits between device and server, intercepting/modifying data (e.g. changing FAIL to PASS). Fix: validate SSL certificates in firmware via certificate pinning.

- Replay attack: Capturing a valid command packet and resending it repeatedly to manipulate records. Fix: add timestamps and nonce values to every API request; reject duplicates.

- Denial of service (DoS): Flooding the ESP32 with packets, crashing its Wi-Fi stack. Fix: implement firmware-level rate limiting; use a router-level firewall.

- Firmware injection: Unprotected OTA updates let an attacker push malicious firmware remotely. Fix: sign OTA packages with a private key and verify the signature before flashing.

Five security improvements:

- Use HTTPS for all Telegram/Google Sheets API communication — encrypts data in transit at no extra hardware cost.

- Strong Wi-Fi password + WPA2/WPA3 on the router; avoid default credentials; enable a router-level firewall.

- API token authentication with short-lived, refreshable tokens; never hardcode permanent credentials — store in encrypted NVS on the ESP32.

- VLAN isolation: place IoT devices on a separate VLAN/guest network so a compromised sensor cannot pivot to attack other resources.

- Disable unused services: turn off Bluetooth if unneeded, disable the serial console in production firmware, and close unused ports to minimize the attack surface.

# Q10. ESP32 with DHT11, Ultrasonic, OLED, Relay, Soil Moisture — Conflicts & Solutions

- DHT11 strict timing: its 1-wire protocol needs precise microsecond timing; any interrupt during the read cycle corrupts data. Fix: disable interrupts (noInterrupts() / portDISABLE_INTERRUPTS()) during the DHT11 read.

- Ultrasonic timing: HC-SR04’s pulseIn() is a blocking call; reading it and DHT11 in the same loop causes one to block the other. Fix: use FreeRTOS tasks, assigning each sensor its own task/timing.

- Relay voltage spike: the relay coil’s back-EMF on switch-off can corrupt ADC readings from the soil moisture sensor on the same power rail. Fix: add a flyback diode (1N4007) across the relay coil in reverse bias.

- OLED I2C bus sharing: multiple I2C devices without proper addressing cause bus collisions. Fix: assign unique addresses (OLED at 0x3C, others at 0x27, etc.) and verify with an I2C bus scanner.

- ADC noise from digital switching: fast switching of relay/digital sensors injects noise into the ESP32 ADC, causing fluctuating soil-moisture readings. Fix: use GPIO34/35 (input-only, no digital switching noise) for the analog sensor, plus a 100 nF capacitor on the ADC pin.

Software solution — dual-core FreeRTOS: assign sensor-reading tasks (DHT11, ultrasonic, soil moisture) to Core 0 and display/relay/communication tasks to Core 1, so fast sensor reads never block on slower I2C display updates.

Suggested pin assignment: DHT11 → GPIO4, HC-SR04 Trig → GPIO5 / Echo → GPIO18, OLED SDA → GPIO21 / SCL → GPIO22, Relay → GPIO23, Soil Moisture → GPIO34 (analog-only). Keep analog and digital pins physically separated.

# Q11. ESP32 Cloud Upload Fails for 12 Hours — Strategy to Prevent Data Loss

- Detect connection failure: check WiFi.status() != WL_CONNECTED or an HTTP timeout (no response within 5 s) before each upload attempt; switch immediately to offline storage mode.

- Store locally in SPIFFS: write each reading to ESP32 internal flash (1–4 MB) as CSV — timestamp,ec,temp,result — one row per reading.

- Timestamp with RTC: use an NTP-synced timestamp cached before going offline, or a DS3231 RTC module, so offline data remains meaningful for analysis.

- Circular buffer for overflow: if SPIFFS nears capacity during a long outage, overwrite the oldest records. (12 hours × 60 readings/hour = 720 rows × ~50 bytes ≈ 36 KB — comfortably within SPIFFS.)

- Batch upload on reconnection: once WiFi.status() == WL_CONNECTED, read stored rows sequentially and upload to Google Sheets/MQTT in batch, oldest record first.

- Delete only after confirmed upload: remove a SPIFFS record only after an HTTP 200 OK; if a batch upload fails partway, retry from the last unconfirmed record.

- Notify the operator: blink a status LED while offline, and send Telegram messages such as “Device went offline at 10:30 AM” and “Back online — uploading 720 stored records.”

Capacity check: 720 readings × 50 bytes ≈ 36 KB — even 30 days of missed data fits comfortably in 1–4 MB SPIFFS without an external SD card.

# Q12. IoT Device Operating 5 Years Without Human Intervention

Hardware changes:

- Industrial-grade components: rated for –40°C to +85°C; capacitors with 10,000+ hour lifespans; ceramic/tantalum instead of electrolytic capacitors.

- External hardware watchdog (e.g. MAX823) that physically resets the board if firmware hangs — independent of a software-only watchdog, which fails if the whole OS crashes.

- Redundant power with UPS: a LiPo battery + solar charger so outages don’t interrupt operation, plus TVS diodes to protect against voltage spikes over the 5-year lifespan.

- IP65 waterproof enclosure with silica gel desiccant packets — moisture ingress is the #1 cause of long-term field-electronics failure.

- Solid-state relays (rated for millions of cycles) instead of mechanical relays (~100,000 cycles), which would wear out over 5 years of continuous operation.

Software changes:

- Watchdog + auto-reboot: call esp_task_wdt_reset() in every task; log every reboot reason to flash for remote diagnosis.

- OTA firmware updates: remotely fix bugs discovered during the 5-year period; keep a backup partition so a failed OTA automatically rolls back.

- Error logging & health monitoring: log all errors/reboots/anomalies to SPIFFS; send a daily health heartbeat to the cloud, alerting if it stops for over 30 minutes; monitor battery voltage, Wi-Fi signal strength, and sensor calibration drift.

# Q13. Can an IoT System Be “Smart” Without AI? Smart vs Intelligent

Yes — IoT can be smart without AI. Smartness means the ability to sense, process, and respond to the environment automatically and autonomously. This does not require AI — rule-based logic, threshold comparisons, and conditional automation all qualify as “smart.”

Examples: - A thermostat that turns on heating below 20°C uses simple threshold logic — no ML, yet universally considered smart. - A milk adulteration tester that flags EC-conductivity thresholds and sends automatic Telegram alerts is fully smart behaviour using only if-else logic, zero AI. - An irrigation system that waters crops only when soil moisture falls below 30% is smart through rule-based automation alone.

Smart vs Intelligent: - Smart = automated, connected, and responsive; senses and acts on data without human intervention, based on pre-defined rules. - Intelligent = learns from experience, adapts behaviour over time, and makes predictions from data — the hallmark of AI/ML systems.

Analogy: A traffic light with a fixed timer is neither smart nor intelligent. One that detects vehicle density and adjusts timing is smart. One that learns traffic patterns over weeks and predicts future congestion is intelligent.

Conclusion: All intelligent IoT systems are smart, but not all smart systems are intelligent — AI adds adaptability and learning as an enhancement to smartness, not a prerequisite for it.

# Q14. Polling vs Interrupt-Based Programming

- Polling: the CPU continuously checks a peripheral’s status in a loop (e.g. “is the button pressed?”), consuming CPU cycles even when nothing is happening.

- Interrupts: a hardware signal immediately pauses the main program to run an ISR the moment an event occurs, then resumes — the CPU is free between events.

When polling is acceptable: events are frequent and predictable, the system has nothing else to do between checks, and timing precision isn’t critical.

When interrupts significantly improve performance: 1. Time-critical events: an emergency-stop button must be detected instantly regardless of what the CPU is doing — a busy polling loop might miss a brief press; an interrupt fires instantly. 2. Power saving: ESP32 can sleep between events and wake only on interrupt (GPIO/timer/RTC) — essential for battery-powered devices (~10 µA sleeping vs ~240 mA active). 3. Pulse counting: flow meters, encoders, or frequency counters need every pulse counted precisely; polling may miss pulses if the loop is slow, while hardware interrupts count every pulse. 4. Multitasking: with multiple peripherals (button + sensor + communication), serial polling is slow and may miss events; interrupts let each peripheral demand attention only when needed.

Applied example: using a hardware interrupt for a TEST button (attachInterrupt(BTN_PIN, runTest, FALLING)) ensures the press is never missed even if the LCD is mid-update or a cloud API call is in progress.

# Q15. Three Design Trade-offs in an IoT Project (Example: Milk Adulteration Tester)

- Cost vs accuracy: chose a generic EC probe module (~Rs. 220) over a lab-grade sensor (~Rs. 800), sacrificing precision (±0.3 mS/cm vs ±0.01 mS/cm) to keep the build affordable for rural users. Justification: for a go/no-go screening tool, ±0.3 accuracy reliably detects water dilution (which shifts EC by 0.5–1.5 mS/cm); paying 4x more for unneeded precision isn’t justified.

- Simplicity vs feature completeness: chose a single EC conductivity measurement over a multi-parameter array (EC + pH + turbidity + urea strip), sacrificing the ability to detect non-ionic adulterants like starch and urea. Justification: more sensors would triple cost/complexity, while EC alone catches the most common adulterants (water, salt, detergent) in ~80% of cases; a secondary strip test handles edge cases.

- Speed vs measurement stability: chose to average 10 ADC readings over 1 second instead of a single instant reading, sacrificing test speed (5 s total vs ~1 s) for stability and noise filtering. Justification: in a field environment with generator/motor interference, a single reading can vary ±0.5 mS/cm from noise; averaging gives ±0.1 mS/cm stability — and 5 seconds is still far faster than any chemical strip test (2–10 minutes).

All three trade-offs are justified: each sacrifices a dimension the application doesn’t critically need (lab-grade precision, multi-parameter detection, or instant speed) in exchange for a dimension it does need (affordability, simplicity, or reliable readings).

# Q16. Flowchart — Local Storage When Wi-Fi Unavailable + Auto Upload on Reconnection

Programs / protocols / I-O used: SPIFFS (local flash storage), Wi-Fi, MQTT/HTTPS (upload), NTP + DS3231 RTC (timestamps), GPIO (status LED).

Flow:

START — Initialize ESP32
      |
      v
Read sensor data (every 60 seconds)
      |
      v
Is Wi-Fi connected? ------ NO ------> Write reading to SPIFFS
      | YES                            (CSV: timestamp,ec,temp,result)
      v                                       |
Upload to cloud (HTTPS / MQTT)                v
      |                              Is storage > 90% full?
      v                                 | YES         | NO
 HTTP 200 OK? --NO--> retry/queue       v             v
      | YES                     Overwrite oldest   Continue normally
      v                         record (circular    (blink LED: offline)
Clear local buffer / mark        buffer)
uploaded records as sent
      |
      v
On reconnection: read SPIFFS oldest-first,
batch upload, delete each record only
after HTTP 200 OK confirmation
      |
      v
Wait 60s — loop back to "Read sensor data"

- Local storage protocol: SPIFFS stores CSV rows; capacity 1–4 MB holds thousands of readings (12 hours × 60 readings/hour × ~50 bytes ≈ 36 KB).

- Upload protocol: HTTPS POST to Google Sheets Apps Script, or MQTT publish to a broker; a record is deleted locally only after server confirmation.

- Timestamps: NTP sync (pool.ntp.org) while online; DS3231 RTC keeps accurate time offline via its own crystal oscillator.

# Q17. Flowchart — ESP32 OTA Firmware Update with Error Handling & Rollback

Flow:

Normal firmware running
      |
      v
Periodic OTA check (every 6 hours)
      |
      v
Wi-Fi OK? --NO--> Log error, retry in 10 minutes
      | YES
      v
GET /firmware/version.txt from update server
      |
      v
New version available? --NO--> Continue normal operation
      | YES
      v
Telegram alert: "OTA update starting..."
      |
      v
Download firmware.bin, stream to OTA partition
      |
      v
Checksum (MD5/SHA256) valid? --NO--> Abort + rollback,
      | YES                          previous partition boots
      v
esp_restart() -> boot new firmware
      |
      v
Self-test passed? --NO--> Rollback: mark invalid + reboot
      | YES                (esp_ota_mark_app_invalid_rollback_and_reboot())
      v                          |
Telegram: "OTA SUCCESS"           v
      |                    Telegram: "OTA FAILED — rolled back"
      v
Resume normal operation

- OTA check trigger: periodic version check every 6 hours; compares server version string against the current firmware version stored in NVS.

- Firmware verification: MD5/SHA256 checksum validated after download, before flashing.

- Automatic rollback: the ESP32 Arduino OTA framework keeps two firmware partitions; if new firmware fails to boot or fails self-test, it automatically restores the previous working version.

- Notifications: Telegram alerts at each key stage (“OTA starting v1.2 → v1.3”, “OTA SUCCESS — rebooting”, “OTA FAILED — rolled back to v1.2”) keep the operator informed remotely.